ARCMON.SYS Arcade monitor device driver v 1.0

=============================================

Disclaimer

----------

This software is without warranty and use of it is entirely at your own risk.

I take no responsiblity for any personal injuries or

damage to equipment resulting from the use or misuse of this software.

General

-------

ARCMON.SYS is a simple DOS Device Driver which supports

VGA Text mode and VGA 320x200 (256 colours) and allows them to be displayed

on an arcade monitor.

ARCMON.SYS is freeware.

You are free to distribute, copy and use the binary and/or source code

of ARCMON.SYS for any non-commerical purpose,

as long as this file is included.

Usage

-----

place the following line in your config.sys

device=arcmon.sys

If possible, have this as the first line of your config.sys,

so the arcade mode is set as quickly as possible

NOTE:It is advisable to create a DOS boot (floppy) disk which does not

contain arcmon.sys in its config.sys.

That way, if you experience problems with ARCMON.SYS, you can boot

from your A: drive and remove the arcmon.sys line from your hard disk's

config.sys easily

To create a boot disk, place a floppy in drive a: and (from C:) type

sys a:

Command Line Parameters

-----------------------

/x <n> - Center screen in x direction

range         : 0-16

default value : 8

example       : device=arcmon.sys /x 4

notes:values 0-7 move the screen to the right, 9-16 move it to the left

/y <n> - Center screen in y direction

range         : 0-16

default value : 8

example       : device=arcmon.sys /y 2

notes:values 0-7 move the screen down, 9-16 move it up

/h <n> - horizontal total

range         : 0-255

default value : 105

example       : device=arcmon.sys /h 110

notes:this defines the horizontal scanrate of the mode produced,

larger numbers cause a lower scanrate, smaller ones a higher scanrate

ALTERING THIS VALUE BY ANY SIGNIFICANT AMOUNT MAY DAMAGE YOUR MONITOR

/v <n> - vertical total

range         : 0-255

default value : 9

example       : device=arcmon.sys /v 6

notes:this defines the vertical refresh rate of the mode produced,

larger numbers cause a lower refresh rate,

smaller ones a higher refresh rate.

ALTERING THIS VALUE BY ANY SIGNIFICANT AMOUNT MAY DAMAGE YOUR MONITOR

NOTES: when using command line parameters there MUST be a space between the

parameter identifier and the value being set.

i.e.

The following will work:

device=arcmon.sys /x 12 /y 4

THE FOLLOWING WILL NOT:

device=arcmon.sys /x12 /y4

Final Notes

-----------

If you're using Win95/98 with a 'doctored' MSDOS.SYS so it boots straight

to DOS.

i.e.

first make MSDOS.SYS visible and editable with:

attrib msdos.sys -s -h -r

Then edit it and set

BootGUI=0

in the [Options] section.

The machine now boots to DOS, but Win95/98 can still be started

with the command

win

I suggest you add the line

Logo=0

to the [Options] section in MSDOS.SYS

This will stop the 'Loading' graphic screen appearing while the device drivers

are being loaded.

Brian Lewis 1999

++++++++++++++++